SECTION 09 51 26

ACOUSTICAL WOOD CEILINGS

WOODWORKS SHAPES FOR DESIGNFLEX GRILLE TEGULAR

PART 1 – GENERAL

  - RELATED DOCUMENTS

Drawings and general conditions of Contract, including General and Supplementary Conditions and Divisions-1 Specification sections apply to work of this section.

  - SUMMARY

- Section Includes

  - Wood ceilings

  - Exposed grid suspension system

  - Wire hangers, fasteners, main runners, cross tees, and wall angle moldings

  - Perimeter Trim

- Related Sections

  - Section 09 53 00 - Acoustical Ceiling Suspension Assemblies

  - Section 09 20 00 - Plaster and Gypsum Board

  - Section 09 22 16 – Non-structural Metal Framing

  - Division 23 (15) - HVAC Air Distribution

  - Division 26 (16) – Electrical Work

- ALTERNATES

  - Prior Approval: Unless otherwise provided for in the Contract documents, proposed product substitutions may be submitted no later than TEN (10) working days prior to the date established for receipt of bids.  Acceptability of a proposed substitution is contingent upon the Architect's review of the proposal for acceptability and approved products will be set forth by the Addenda.  If included in a Bid are substitute products that have not been pre-approved by the architect and included in the Addenda, the originally specified products shall be provided without additional compensation.

  - Submittals that do not provide adequate data for the product evaluation will not be considered.  The proposed substitution must meet all requirements of this section, including but not necessarily limited to, the following: Single source materials suppliers; Panel design, size, composition, color, and finish; Suspension system component profiles and sizes; Compliance with the referenced standards.

  - REFERENCES

- American Society for Testing and Materials (ASTM):

  - ASTM A 1008 Standard Specification for Steel, Sheet, Cold Rolled, Carbon, Structural, High-Strength Low-Alloy and High-Strength Low-Alloy with Improved Formability

  - ASTM A 641 Standard Specification for Zinc-Coated (Galvanized) Carbon Steel Wire

  - ASTM A 653 Standard Specification for Steel Sheet, Zinc-Coated (Galvanized) by the Hot-Dip Process

  - ASTM C 635 Standard Specification for Metal Suspension Systems for Acoustical Tile and Lay-in Panel Ceilings

  - ASTM C 636 Recommended Practice for Installation of Metal Ceiling Suspension Systems for Acoustical Tile and Lay-in Panels

  - ASTM E 84 Standard Test Method for Surface Burning Characteristics of Building Materials

  - ASTM E 580 Installation of Metal Suspension Systems in Areas Requiring Moderate Seismic Restraint

  - ASTM C 423 Sound Absorption and Sound Absorption Coefficients by the Reverberation Room Method.

  - ASTM E 1264 Classification for Acoustical Ceiling Products

- International Building Code

- ASHRAE Standard 62.1-2004, Ventilation for Acceptable Indoor Air Quality

- NFPA 70 National Electrical Code

- ASCE 7 American Society of Civil Engineers, Minimum Design Loads for Buildings and Other Structures

- International Code Council-Evaluation Services - AC 156 Acceptance Criteria for Seismic Qualification Testing of Non-structural Components

- International Code Council-Evaluation Services Report - Seismic Engineer Report

  - ESR 1308 - Armstrong Suspension Systems

- California Air Resource Board (CARB) compliant

- LEED - Leadership in Energy and Environmental Design is a set of rating systems for the design, construction, operation, and maintenance of green buildings

- Clean Rooms up to ISO Class 5 (Class 100)

  - SUBMITTALS

- Shop Drawings: Layout and details of ceilings. Show locations of items that are to be coordinated with or supported by the ceilings.

- Installation Instructions: Submit manufacturer’s installation instructions as referenced in Part three, Installation.

- Product Data: Submit manufacturer’s technical data for each type of ceiling unit and suspension system required.

- Samples: Veneer Finishes – Clear satin finish.

- Certifications: Manufacturer’s certifications that products comply with specified requirements, including laboratory reports showing compliance with specified tests and standards.

- Non-Conformance: All products not conforming to the requirements of this specification and or the manufacturer’s published values are to be disposed. The Contractor performing the work will replace with approved product at their expense.

  - QUALITY ASSURANCE

- Single-Source Responsibility: Provide acoustical panel units and grid components by a single manufacturer to ensure fit and function.

- Installer Qualifications:  Company specializing in performing specified work type, a minimum of three years of documented experience, and approved by the manufacturer.

- Fire Performance Characteristics: Identify acoustical ceiling components with appropriate markings of applicable testing and inspecting organization.

- Surface Burning Characteristics: Tested per ASTM E 84 and complying with ASTM E 1264 Classification.

- HPVA (Hardwood Plywood and Veneer Association) certification and audit program per ASTM E-84 tunnel test.

- Woodworking Standards: Manufacturer must comply with specified provisions of Architectural Woodworking Institute quality standards.

- Coordination of Work: Coordinate ceiling work with installers of related work including, but not limited to building insulation, gypsum board, light fixtures, mechanical systems, electrical systems, and sprinklers.

  - DELIVERY, STORAGE, AND HANDLING

- Store ceiling components in a dry interior location in their cartons prior to installation to avoid damage. Store cartons in a flat, horizontal position. The protectors between the panels should not be removed until installation.

- Do not store in unconditioned spaces with humidity greater than 55 percent or lower than 25 percent relative humidity and temperatures lower than 50 degrees F or greater than 86 degrees F. Panels must not be exposed to extreme temperatures, for example, close to a heating source or near a window with direct sunlight.

- Handle ceiling units carefully to avoid chipped edges or damage to units in any way.

  - PROJECT CONDITIONS

- Wood ceiling materials should be permitted to reach room temperature and have a stabilized moisture content for a minimum of 72 hours before installation. (Remove plastic wrap to allow panels to climatize).

- The wood panels should not be installed in spaces where the temperature or humidity conditions vary from the temperatures and conditions that will be normal in the occupied space.

- As interior finish products, the solid wood panels are designed for installation in temperature conditions between 50 degrees F and 86 degrees F, in spaces where the building is enclosed, and HVAC systems are functioning and will be in continuous operation. Relative humidity should not fall below 25 percent or exceed 55 percent.

  - WARRANTY

- Submit a written warranty executed by the manufacturer, agreeing to repair or replace panels that fail within the warranty period. Failures include, but are not limited to:

  - Ceiling: Defects in materials or factory workmanship.

  - Grid System: Rusting and manufacturing defects.

- Warranty Period:

  - Ceiling: One (1) year from date of installation.

  - Grid: Ten years from date of installation.

- The Warranty shall not deprive the owner of other rights the owner may have under other provisions of the Contract Documents and will be in addition to and run concurrent with other warranties made by the Contractor under the requirements of the Contract Documents.

  - MAINTENANCE

- Extra Materials: Deliver extra materials to Owner.  Furnish extra materials described below that match products installed.  Packaged with protective covering for storage and identified with appropriate labels.

  - Ceiling Units:  Furnish quality of full-size units equal to 2.0 percent of amount installed.

  - Exposed Suspension System Components:  Furnish quantity of each exposed suspension component equal to 1.0 percent of amount installed.

PART 2 – PRODUCTS

Attention Design Professional: Please edit Part 2 based on your project needs. Select product attributes and acceptable product item (s) that fit with the requirements of your project. Delete all items from the specification that do not relate to your project needs. Please refer to the Armstrong website for additional ceilings, suspension systems, perimeter trim options, and accessories. The related guide specifications for each of these items are available on the Armstrong website.

  - MANUFACTURERS

  - Ceiling Panels:

    - Armstrong World Industries, Inc.

  - Suspension Systems:

    - Armstrong World Industries, Inc.

  - ACOUSTICAL WOOD CEILINGS / SUSPENDED WOOD CEILINGS

  - Acoustical Wood Ceilings

    - Surface Texture: Smooth

    - Composition:  Fire-retardant medium-density fiberboard with face-cut veneers

    - Finish: Clear satin finish. All Finishes are part of Sustain Portfolio

      - Plain Slice White Maple (NWM)

      - Plain Slice White Ash (NWA)

      - Vertical Grain Fir (NVF)

      - Rift White Oak (NRO)

      - Plain Slice White Oak (NOK)

      - Plain Slice Cherry (NPC)

      - Quartered Mahogany (NQM)

      - Quartered Sapele (NQS)

      - Quartered Walnut (NQW)

      - Plain Slice Walnut (NWN)

    - Design Options & Material IDs:

      - 5577F09T01_ _ _: Triangle with Vertical Slats 24 x 33 x 48”

      - 5577F09T02_ _ _: Triangle with Horizontal Slats 24 x 33 x 48”

      - 5577F09T03_ _ _: Right Triangle with Vertical Slats 24 x 34 x 24”

      - 5577F09T04_ _ _: Right Triangle with Horizontal Slats 24 x 34 x 24”

      - 5577F09P01_ _ _: Left Parallelogram with Vertical Slats 24 x 33 x 48”

      - 5577F09P02_ _ _: Left Parallelogram with Horizontal Slats 24 x 33 x 48”

      - 5577F09P03_ _ _: Right Parallelogram with Vertical Slats 24 x 33 x 48”

      - 5577F09P04_ _ _: Right Parallelogram with Horizontal Slats 24 x 33 x 48”

    - Acoustical Performance Infill Options:

      - Calla Shapes Lay-in panels:

        - 8260T01 (Triangle), NRC .60

        - 8260T03 (Right Triangle) NRC .60

        - 8260P01 (Left Parallelogram) NRC .60

        - 8260P03 (Right Parallelogram) NRC .60

    - Flame Spread:  ASTM E 1264; Class A

    - Basis of Design: WOODWORKS Shapes for DesignFlex Linear Grille as manufactured by Armstrong World Industries, Inc.

    - Ceiling Accessories:

      - Axiom® Perimeter Corner Slip- item PCC

      - Perimeter Angle Slip- item PAC

      - 2” Beam End Retaining Clip- item BERC2

      - T-Bar Connector Clip- item FXTBC

      - WoodWorks End Paint (Quart Size) - item 6776QT1 _ _ _ (Finish Suffix)

      - WoodWorks End Paint (Gallon Size) - item 6776GAL1_ _ _ (Finish Suffix)

    - Substitutions: Refer to Alternates in Part 1.

  - METAL SUSPENSION SYSTEMS

- Components: Main beams and cross tees, base metal and end detail, fabricated from commercial quality hot dipped galvanized steel complying with ASTM A 653.  Main beams and cross tees are double-web steel construction exposed flange design.  Exposed surfaces chemically cleansed, capping prefinished galvanized steel in baked polyester paint.  Main beams and cross tees shall have rotary stitching.

  - Structural Classification:  ASTM C 635 Heavy Duty.

  - Basis of Design:

    - Suprafine XL 9/16" Exposed Tee 12’ Main Beam - item 7501BL as manufactured by Armstrong World Industries, Inc.

    - Suprafine XL 9/16” Exposed Tee 2’ Cross Tee - item XL7520BL as manufactured by Armstrong World Industries, Inc.

    - Suprafine 45° Base Cross Tee- 24” Main Beam Spacing – item XM754524_ _  as manufactured by Armstrong World Industries, Inc.

    - Suprafine 45° Left Angle Bracket- item 75AB45L

    - Suprafine 45° Right Angle Bracket- item 75AB45R

    - Suprafine 45° Double Angle Bracket- item 75AB45D

    - Suprafine 45° Corner Bracket- item 75CB45

  - Substitutions: Refer to Alternates in Part 1.

- Attachment Devices: Size for five times design load indicated in ASTM C 635, Table 1, Direct Hung unless otherwise indicated.

- Wire for Hangers and Ties: ASTM A 641, Class 1 zinc coating, soft annealed, with a yield stress load of at least time three design load, but not less than 12 gauge.

- Suspension System Accessories, Edge Moldings and Trim as manufactured by Armstrong World Industries, Inc.

  - 7/8” Wall Molding - item7800BL

  - Angle Molding- item 7804

  - Axiom Straight Trim - Recommend in Black 6” and up – AX_VESTR_ _ _ (Finish Suffix)

  - Axiom Curved Trim - Recommend in Black 6” and up – AX_VECUR_ _ _ (Finish Suffix)

  - WoodWorks Trim Aluminum Substrate - 4”- item 5344BL

  - WoodWorks Trim Aluminum Substrate – 6” – item 5346BL

PART 3 – EXECUTION

3.1 EXAMINATION

- Do not proceed with installation until all wet work such as concrete, terrazzo, plastering and painting has been completed and thoroughly dried out.

- Proper designs for both supply air and return air, maintenance of the HVAC filters and building interior space are essential to minimize soiling. Before starting the HVAC system, make sure supply air is properly filtered and the building interior is free of construction dust.

3.2 PREPARATION

- Measure each ceiling area and establish layout of acoustical units to balance border widths at opposite edges of each ceiling.  Avoid use of less than half width units at borders and comply with reflected ceiling plans.  Coordinate panel layout with mechanical and electrical fixtures.

- WoodWorks ceiling materials should be permitted to reach room temperature and have a stabilized moisture content for a minimum of 72 hours before installation. (Remove plastic wrap to allow panels to climatize).

3.3 INSTALLATION

- Interior WoodWorks products, the solid wood panels are designed for installation in temperature conditions between 50 degrees F and 86 degrees F, in spaces where the building is enclosed, and HVAC systems are functioning and will be in continuous operation. Relative humidity should not fall below 25 percent or exceed 55 percent.

- Install suspension system and panels in compliance with ASTM C636, ASTM E580, with the approval of the authorities having jurisdiction, and in accordance with the manufacturer’s Installation Instructions.

3.4 ADJUSTING AND CLEANING

- Replace damaged and broken panels.

- Clean exposed surfaces of acoustical ceilings, including trim, edge moldings, and suspension members. Comply with manufacturer's instructions for cleaning and touch up of minor finish damage.