Architectural Components Group, Inc.

521 George St.

Marshfield, MO 65706

417-869-6777  /  417-869-7888 fax

info@acgiwood.com / acgiwood.com

ARCHITECTURAL SPECIFICATIONS FOR

SUSPENSION SYSTEM SERIES 1 WOOD CEILING and WALL

[Project Name]

[Project Location]

PART 1 — GENERAL

1.1 Description of Work:

- Work Included: Provide Architectural Components Group, Inc. Suspension System Series 1 SS1 ceiling and wall where shown on architectural drawings, and as specified in this specification.

1.2 Quality Assurance:

- Architectural Components Group Suspension System Series 1 ceiling and wall products for this project shall maintain the quality as instituted by the architect or A.W.I.

- Design Criteria: Manufacture of Suspension System SS1-xxxx-C shall be installed true and plumb to within manufacturing tolerance of 1/8” within 8’ of length.

- Product Construction: Wood shall be kiln dried to 10%. Cracking, checking and warpage of members will not be acceptable.

1.3 Submittals:

- Technical Data: Submit manufacturer’s data and installation instructions.

- Shop Drawings: Submit 4 copies of shop drawings showing all areas involved, attachment conditions and perimeter circumstances.

- Submittal Samples: Submit 1 product sample for approval by architect.

1.4 DELIVERY, STORAGE AND HANDLING

- Material must be stored and installed only in secured ambient environment (humidity min. 25% -max. 55%, temperature not to exceed 86 deg.)

- Windows, Doors and all wet-work must be completed before un-packaging and installation. Handle carefully to avoid damaging.

PART 2 — PRODUCTS

2.1 Acceptable Manufacturer:

Architectural Components Group, Inc., 521 George Street, Marshfield, MO 65706 Phone (417) 869-6777	Fax (417) 869-7888

2.2 Product Type

- Product: Suspension System Series 1 wood ceiling & wall shall be product: SS1-xxxx-C

- Size: Suspension System Series 1 Flat Panels shall be provided in a panel size of: ___” x ___” (e.g. 24” x 24”) and 3/4” thick.

Nomenclature for SS1-2424-C:

SS1 = Suspension System Series 1

2424 = a panel 24” x 24”

C = 3/4” thick substrate

- Specie: System shall consist of specie wood veneer [e.g., cherry plain-sliced, book-matched].

- Perforations: Panels shall or shall not have perforations. If perforations, then Xmm holes Xmm O.C. with perforation pattern parallel, offset, or xx/xx pattern. Some available perforation patterns are: 6mm or 8mm Holes, 16mm or 32mm O.C.

- Fixture areas: Fixture cut-outs are available factory made or field cut. Perforated panels shall have factory-made non-perforated areas to allow for 3”, 6”, or  9” fixture cut-outs. Non-perforated areas are to be round or square. Penetration areas shall be centered within the panel.

- Fire Rating: Wood panels shall achieve a Class I(A) or Class III(C) fire rating.

- Finish: Wood veneer finish shall utilize ACGI clear finish with satin sheen or specify stain to match architect sample.

- Wood veneer shall or shall not be FSC certified. The core material shall or shall not be FSC certified. If FSC, Chain of Custody shall or shall not be provided.

- Substrate material shall or shall not be manufactured with no added urea formaldehyde.

- Attachment System: Suspension System wood ceiling shall be suspended according to manufacturer’s suggested method of suspension as per the design details provided in the plans.

- Suspension:

  - Ceiling Panels shall be suspended using the SS1 type suspension system as indicated by ACGI. Unless otherwise stated, all acoustical grid will be 15/16” heavy duty grid with main runners on 2’ centers.

  - Wall Panels shall be attached with standard Z-clips.

PART 3 — EXECUTION

3.1 Ceiling Installation

- Suspension System wood ceiling shall be handled and installed with care in order to prevent surface and structure damage. Field cutting shall be kept to a minimum and performed as recommended by manufacturer.

- The ceiling system shall be suspended by 15/16” HD T-Grid with main runners on 2’ centers and cross T’s every 2’.

- The contractor shall follow the manufacturer’s installation instructions as provided by ACGI.

3.2 Installation

- Suspension System wood wall shall be handled and installed with care in order to prevent surface and structure damage. Field cutting shall be kept to a minimum and performed as recommended by manufacturer.

- Suspension System wood wall shall be installed using standard Z-clips. SS1 wall panel sizes shall be up to 5’ wide x 12’ length [limited by veneer length].

- The contractor shall follow the manufacturer’s installation instructions as provided by ACGI.

PART 4 — WARRANTY

4.1 Product Warranty

- Architectural Components Group, Inc. shall warranty the Suspension System Series 1 wood ceiling against manufacturer’s defects up to one year. Changes in finish or dimensions due to ultra violet light, excessive temperature or humidity conditions [see 1.4.A above] and/or abuse of any kind shall void any warranties from ACGI.

- Install conditions for the area should remain for the proper occupied conditions of humidity and temperature (humidity min. 25% - max. 55%, temperature not to exceed 86 degrees).

- The contractor shall warranty for one year all work from final acceptance of completed work.